RTL_EXERCISE_2

SPI Communication

Design Specification

| Author | L03_Group 2 |
| --- | --- |
| Date | 2026/05/17 |
| Version | 2.0 |
| Status | RTL-Aligned Revision |

# 1. Interface

The top-level module spi_communication instantiates one spi_master and one spi_slave. The complete signal list is shown in Table 1. Note that SS is 3-bit [2:0], reflecting the decoded slave-select width in the RTL. M_SELECT_SS[2:0] is a dedicated slave-address port, separate from M_INPUT[7:0].

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| REFCLK | 1 | In | Reference clock for the complete design. All sequential logic is synchronised to this clock. |
| RST_N | 1 | In | Active-low asynchronous reset. Deasserted high to enable normal operation. |
| M_INPUT[7:0] | 8 | In | Master TX data byte. Loaded into the master transmit register when CNTL=01 and M_READY=1. |
| M_SELECT_SS[2:0] | 3 | In | Slave address (0–7). Stored into selected_ss when CNTL=10. Active-low one-hot decode is computed as ~(3'b001 << M_SELECT_SS). |
| M_CNTL[1:0] | 2 | In | Master command: 00=no-op, 01=load TX data, 10=store slave address, 11=start transfer. |
| M_OUTPUT[7:0] | 8 | Out | Master received byte from MISO. Updated after the 8th bit is sampled. |
| M_READY | 1 | Out | High in ST_IDLE, low during ST_TRANSFER and ST_WAIT_RELEASE. |
| S_INPUT[7:0] | 8 | In | Slave transmit byte. Loaded into tx_shift when S_LOAD=1 and S_READY=1. |
| S_LOAD | 1 | In | Synchronous load enable for slave TX register. Active high; has effect only when S_READY=1. |
| S_OUTPUT[7:0] | 8 | Out | Slave received byte from MOSI. Updated after the 8th bit is sampled. |
| S_READY | 1 | Out | High when CS is inactive AND slave is not active. Combinational: (!cs_active) && (active==0). |
| MOSI | 1 | Out | Master-to-slave serial data. Driven from tx_data[bit_cnt] MSB-first. |
| MISO | 1 | Out | Slave-to-master serial data. Exposed at top level for observation. Driven to 0 when CS is inactive. |
| SCLK | 1 | Out | SPI clock generated by master. LOW in idle; toggled during ST_TRANSFER. |
| SS[2:0] | 3 | Out | Active-low slave select (3-bit). SS[0] selects the integrated slave. 3'h7 when inactive. |

Table 1: Signal description of spi_communication

# 2. Functional Description

## 2.1 SPI Protocol

The design implements SPI Mode 0 (CPOL=0, CPHA=0): SCLK is LOW while idle, data is captured on the rising SCLK edge and updated on the falling edge. Transfer width is 8 bits, MSB-first.

## 2.2 Master Command Decode

CNTL is evaluated each REFCLK cycle in ST_IDLE:

| CNTL | Name | Behaviour |
| --- | --- | --- |
| 2'b00 | No-op | No action. Master stays in ST_IDLE. |
| 2'b01 | Load TX data | tx_data <= M_INPUT[7:0]. No SS assertion, no SCLK. |
| 2'b10 | Store slave address | selected_ss <= ~(3'b001 << M_SELECT_SS). SS remains 3'h7. Valid for M_SELECT_SS in range 0-7 (always true for 3-bit input). |
| 2'b11 | Start transfer | Transition to ST_TRANSFER. READY=0, SS=selected_ss, MOSI=tx_data[7], bit_cnt=7. |

Table 2: Master CNTL command decode

## 2.3 Master State Machine

The master has three states: ST_IDLE, ST_TRANSFER, ST_WAIT_RELEASE.

| State | Description |
| --- | --- |
| ST_IDLE | Accepts CNTL commands. READY=1, SCLK=0, MOSI=0, SS=3'h7. |
| ST_TRANSFER | Performs 8-bit SPI transaction. sclk_level toggles between 0 (falling, prepare MOSI) and 1 (rising, sample MISO). After bit 0 falling edge, transitions to ST_WAIT_RELEASE and deasserts SS. |
| ST_WAIT_RELEASE | Holds READY=0, SS=3'h7 while CNTL==2'b11. Returns to ST_IDLE and asserts READY=1 once CNTL is released. |

Table 3: Master state machine

## 2.4 Transfer Timing Detail

In ST_TRANSFER, each REFCLK cycle toggles sclk_level:

- sclk_level=0 → 1: SCLK rises. rx_shift[bit_cnt] <= MISO. If bit_cnt==0, latch data_out = {rx_shift[7:1], MISO}.

- sclk_level=1 → 0: SCLK falls. If bit_cnt==0, transition to ST_WAIT_RELEASE, MOSI=0, SS=3'h7. Otherwise bit_cnt--, MOSI=tx_data[bit_cnt-1].

Total transfer duration: 16 REFCLK cycles (8 rising + 8 falling edges). SS is asserted from the first REFCLK of ST_TRANSFER and deasserted on the falling edge after bit 0.

## 2.5 Slave Operation

The slave is clocked by REFCLK with asynchronous active-low reset RST_N.

- SCLK rising-edge detection: sclk_d is updated each REFCLK; sclk_rise = (SCLK==1) && (sclk_d==0).

- On sclk_rise: rx_shift[bit_cnt] <= MOSI. When bit_cnt==0, data_out <= {rx_shift[7:1], MOSI} and bit_cnt resets to 3'd7.

- MISO is combinational: cs_active ? tx_shift[bit_cnt] : 1'b0. When CS is inactive, MISO is driven to logic 0.

- S_LOAD takes effect only when S_READY==1 (i.e., LOAD && READY condition). Loading during an active CS is ignored.

- S_READY = (!cs_active) && (active==0). Both conditions must be true simultaneously.

## 2.6 Slave Select Decode

The master computes SS as: selected_ss = ~(3'b001 << M_SELECT_SS[2:0]). Because M_SELECT_SS is 3-bit, all input values (0–7) are in-range. Value 7 maps to SS=3'h7 (all bits low→inactive), which prevents accidental slave selection when address 7 is chosen. The following table shows the expected SS output per address:

| M_SELECT_SS | Shift Amount | SS[2:0] (active-low) |
| --- | --- | --- |
| 0 | 001 << 0 = 001 | 3'b110 = 3'h6 |
| 1 | 001 << 1 = 010 | 3'b101 = 3'h5 |
| 2 | 001 << 2 = 100 | 3'b011 = 3'h3 |
| 3 | 001 << 3 = 000 (overflow) | 3'b111 = 3'h7 |
| 4–6 | Similar overflow | 3'h7 (all inactive) |
| 7 | 001 << 7 = 000 (overflow) | 3'h7 (all inactive) |

Table 4: Slave-select decode table

Note: The integrated slave in spi_communication is connected to SS[0] (cs_slave0 = ss_w[0]). Slave 0 is selected when SS[2:0]=3'h6 (SS[0]=0).

# 3. Internal Implementation

## 3.1 Block Hierarchy

| Module | Instance | Function |
| --- | --- | --- |
| spi_communication | — | Top-level wrapper. Connects master, slave, and exposes SPI observation pins. |
| spi_master | u_spi_master | Generates SCLK, drives MOSI MSB-first, samples MISO on rising edges, decodes slave address, controls SS. |
| spi_slave | u_spi_slave | Synchronously loads TX data, detects SCLK rising edge via sclk_d, samples MOSI, drives MISO combinationally. |

Table 5: Module hierarchy

## 3.2 Key Internal Variables

| Variable | Module | Description |
| --- | --- | --- |
| state [1:0] | spi_master | Current FSM state (ST_IDLE=0, ST_TRANSFER=1, ST_WAIT_RELEASE=2). |
| tx_data [7:0] | spi_master | Master transmit byte, loaded by CNTL=01. |
| rx_shift [7:0] | spi_master | Shift register assembling the received byte from MISO. |
| selected_ss [2:0] | spi_master | Stored one-hot active-low slave select, updated by CNTL=10. |
| bit_cnt [2:0] | spi_master / spi_slave | Bit index, counts down from 7 to 0 during transfer. |
| sclk_level | spi_master | Internal SCLK phase flag: 0=falling (prepare), 1=rising (sample). |
| tx_shift [7:0] | spi_slave | Slave transmit shift register. Loaded from data_in when LOAD && READY. |
| active | spi_slave | Slave transaction active flag. Set when cs_active=1, cleared when cs_active=0 or bit_cnt reaches 0. |
| sclk_d | spi_slave | One-cycle delayed SCLK used for rising-edge detection. |
| cs_active | spi_slave | Wire: cs_active = (CS==0). Used for all conditional logic. |

Table 6: Key internal variables

## 3.3 Reset Behaviour

| Register | Reset Value |
| --- | --- |
| Master: state | ST_IDLE (2'd0) |
| Master: tx_data, rx_shift, data_out | 8'h00 |
| Master: selected_ss, SS | 3'h7 (all slaves deselected) |
| Master: bit_cnt | 3'd7 |
| Master: sclk_level, SCLK, MOSI | 1'b0 |
| Master: READY | 1'b1 |
| Slave: tx_shift, rx_shift, data_out | 8'h00 |
| Slave: bit_cnt | 3'd7 |
| Slave: active, sclk_d | 1'b0 |

Table 7: Reset values

# 4. Revision History

| Date | Author | Version | Description |
| --- | --- | --- | --- |
| 2026/05/17 | L03_Group 2 | v0.1 | Created top-level signal list and SPI communication interface. |
| 2026/05/17 | L03_Group 2 | v0.2 | Implemented ASIC-friendly reset, synchronous slave LOAD, master SS control, 8-bit Mode-0 MSB-first transfer. |
| 2026/05/17 | L03_Group 2 | v0.3 | Added self-checking testbenches with PASS/FAIL logs and waveform dumps. |
| 2026/05/17 | L03_Group 2 | v1.0 | Finalized Genus synthesis at 2.5 ns, Fmax 400 MHz, Slack 0 ps, TNS 0.0. |
| 2026/05/17 | L03_Group 2 | v2.0 | Design spec revised to match RTL: 3-bit SS, M_SELECT_SS port, MISO=0 when CS inactive, LOAD guard condition, corrected SS decode table. |